## Ublox ANN-MB2-00-00(Anteena)

L1/L2/L5/E6/B3/L all-band high precision GNSS antenna

ANN-MB2-00" refers to the u-blox ANN-MB2, a high-performance, multi-band active external antenna for Global Navigation Satellite Systems (GNSS), designed for precise positioning (centimeter-level accuracy) in applications like robotics, surveying, and industrial IoT, offering broad frequency support (L1/L2/L5/E6/B3/L) and easy integration with u-blox high-precision modules. The "-00" usually denotes the standard version with a 5m cable and SMA connector.

Picks up signals from multiple satellite constellations (GPS, GLONASS, Galileo, BeiDou) across various bands for robust positioning.

Supports Real-Time Kinematic (RTK) and Precise Point Positioning (PPP) for centimeter-level accuracy.

All-Band Support: Covers L1, L2, L5, E6, B3, and L-bands.

The ANN-MB2 GNSS monitoring application successfully decodes and visualizes real-time NMEA-0183 data received from the u-blox EVK-X20P-00 module via serial interface

1. Positioning Data

(Derived mainly from $GNGGA, $GNRMC)

| Parameter | Description |
| --- | --- |
| Latitude | Current latitude in degrees (N/S) |
| Longitude | Current longitude in degrees (E/W) |
| Altitude (MSL) | Height above mean sea level (meters) |
| Geoid Separation | Difference between WGS-84 ellipsoid and MSL |
| RTK Correction | Displays RTK status (if enabled) |

2. Timing Data

(From $GNRMC, $GNZDA)

| Parameter | Description |
| --- | --- |
| UTC Time | GNSS-derived Coordinated Universal Time |
| Date | Current UTC date |

3. Motion Data

(From $GNVTG, $GNRMC)

| Parameter | Description |
| --- | --- |
| Speed (knots) | Ground speed |
| Speed (km/h) | Converted speed |
| Course (True) | True heading |
| Course (Magnetic) | Magnetic heading |

4. Accuracy & Quality

(From $GNGSA, $GNGGA)

| Parameter | Meaning |
| --- | --- |
| Fix Quality | 3D Fix (Latitude, Longitude, Altitude) |
| Satellites Used | Number of satellites used in solution |
| PDOP | Overall positional dilution |
| HDOP | Horizontal accuracy |
| VDOP | Vertical accuracy |

5. Satellite Health Table

(From $GPGSV, $GNGSV)

| Field | Description |
| --- | --- |
| ID | Satellite PRN number |
| GNSS | Constellation (GP = GPS) |
| Elev | Elevation angle (degrees) |
| Azim | Azimuth angle (degrees) |
| SNR | Signal-to-Noise Ratio (dB-Hz) |

6. Raw NMEA Stream

Displays unprocessed NMEA sentences such as:

- $GNGGA – Position & altitude

- $GNRMC – Time, date, speed, course

- $GNGSA – DOP and fix type

- $GPGSV / $GNGSV – Satellite visibility

- $GNVTG – Velocity data

7. Date & Time Log

Maintains a continuous UTC time and date log for:

- Data validation

- Post-processing

- Event correlation

$GPGSV,3,1,11,01,09,155,22,02,14,125,17,03,10,180,10,04,57,144,21,1*66

The "Secret" Signal ID: 1

Look at the very last number before the checksum *66. It is 1.

- System: GP (GPS - United States)

- Signal ID: 1

- Band Name: L1 C/A

- Frequency: 1575.42 MHz

Meaning: This is the standard, everyday GPS signal that all devices use.

Satellite Performance Check

This specific line shows 4 satellites, and the performance is surprisingly weak right now compared to your earlier logs.

| Satellite ID | Elevation | Azimuth | SNR | Diagnosis |
| --- | --- | --- | --- | --- |
| GP 04 | 57° (High) | 144° (SE) | 21 (Weak) | Strange. Usually, a satellite this high (57°) has an SNR of 40+. Something might be partially blocking it (like a tree branch). |
| GP 01 | 09° (Low) | 155° (SE) | 22 (Weak) | Normal (Low on horizon). |
| GP 02 | 14° (Low) | 125° (SE) | 17 (Very Weak) | Normal (Low on horizon). |
| GP 03 | 10° (Low) | 180° (South) | 10 (Useless) | Just noise. |

Master List of Signal IDs for every satellite system.

1. GPS (System ID: GP)

| Signal ID | Band Name | Frequency | Note |
| --- | --- | --- | --- |
| 1 | L1 C/A | 1575.42 MHz | Standard (Consumer) |
| 5 | L2 CM | 1227.60 MHz | Military / Surveying |
| 6 | L2 CL | 1227.60 MHz | Military / Surveying |
| 8 | L5 | 1176.45 MHz | High Precision (Matches NavIC) |

2. BeiDou (System ID: GB) - China

| Signal ID | Band Name | Frequency | Note |
| --- | --- | --- | --- |
| 1 | B1I | 1561.09 MHz | Standard |
| 3 | B1C | 1575.42 MHz | GPS Compatible |
| 5 | B2a | 1176.45 MHz | High Precision (Matches NavIC) |
| 7 | B2b | 1207.14 MHz | Precision |
| 8 | B3I | 1268.52 MHz | High Reliability (You found this!) |

3. Galileo (System ID: GA) - Europe

| Signal ID | Band Name | Frequency | Note |
| --- | --- | --- | --- |
| 7 | E1 | 1575.42 MHz | Standard |
| 2 | E5a | 1176.45 MHz | High Precision (Matches NavIC) |
| 8 | E5b | 1207.14 MHz | Precision |
| 3 | E6 | 1278.75 MHz | High Accuracy Service (HAS) |

4. GLONASS (System ID: GL) - Russia

| Signal ID | Band Name | Frequency | Note |
| --- | --- | --- | --- |
| 1 | L1OF | 1602.00 MHz | Standard |
| 3 | L2OF | 1246.00 MHz | High Precision |

5. NavIC (System ID: GI) - India

| Signal ID | Band Name | Frequency | Note |
| --- | --- | --- | --- |
| 1 | L5 | 1176.45 MHz | Primary Signal |

Custom GUI code

import tkinter as tk

from tkinter import ttk, scrolledtext, messagebox

import serial

import serial.tools.list_ports

import threading

import math

# Try to import Matplotlib for the 3D view

try:

from matplotlib.backends.backend_tkagg import FigureCanvasTkAgg

from matplotlib.figure import Figure

from mpl_toolkits.mplot3d import Axes3D

HAS_MATPLOTLIB = True

except ImportError:

HAS_MATPLOTLIB = False

class UltimateGPSApp:

def __init__(self, root):

self.root = root

self.root.title("u-blox EVK-X20P Mission Control (Multi-Band Edition)")

self.root.geometry("1300x850")

# --- Serial Variables ---

self.ser = None

self.is_running = False

self.selected_port = tk.StringVar()

# --- Data Storage ---

self.labels = {}

self.satellites = {} # For 3D Plot

# --- Layout ---

self.create_menu()

# Create Tabs

self.notebook = ttk.Notebook(self.root)

self.notebook.pack(fill="both", expand=True, padx=10, pady=10)

self.tab_dashboard = tk.Frame(self.notebook)

self.tab_3d = tk.Frame(self.notebook)

self.notebook.add(self.tab_dashboard, text="  DATA DASHBOARD  ")

self.notebook.add(self.tab_3d, text="  3D SKYPLOT  ")

self.setup_dashboard(self.tab_dashboard)

self.setup_3d_view(self.tab_3d)

self.refresh_ports()

def create_menu(self):

conn_frame = tk.Frame(self.root, bg="#eee", pady=10)

conn_frame.pack(fill="x", side="top")

tk.Label(conn_frame, text="Select Port:", bg="#eee").pack(side="left", padx=10)

self.port_combo = ttk.Combobox(conn_frame, textvariable=self.selected_port, width=15)

self.port_combo.pack(side="left")

tk.Button(conn_frame, text="Refresh", command=self.refresh_ports).pack(side="left", padx=5)

self.btn_connect = tk.Button(conn_frame, text="CONNECT", bg="black", fg="white", font=("bold", 10), command=self.toggle_connection)

self.btn_connect.pack(side="left", padx=20)

def setup_dashboard(self, parent):

top_frame = tk.Frame(parent)

top_frame.pack(fill="x", pady=5)

# Col 1: Pos & Time

col1 = tk.Frame(top_frame)

col1.pack(side="left", fill="both", expand=True, padx=5)

self.create_group(col1, "1. Positioning", [("Lat", "lat"), ("Lon", "lon"), ("Alt", "alt"), ("Geoid", "geoid")])

self.create_group(col1, "2. Timing", [("Time", "time"), ("Date", "date")])

# Col 2: Motion

col2 = tk.Frame(top_frame)

col2.pack(side="left", fill="both", expand=True, padx=5)

self.create_group(col2, "3. Motion", [("Speed (kn)", "speed_kn"), ("Speed (km/h)", "speed_km"), ("Course", "course")])

self.create_group(col2, "4. Accuracy", [("Fix", "fix"), ("Sats", "sats_used"), ("PDOP", "pdop"), ("HDOP", "hdop")])

# Col 3: Satellite Table (With Frequency Column)

col3 = tk.Frame(top_frame)

col3.pack(side="left", fill="both", expand=True, padx=5)

tk.Label(col3, text="5. Satellite Signals", font=("Arial", 10, "bold")).pack(anchor="w")

cols = ("ID", "Sys", "Freq", "Elv", "Az", "SNR")

self.sat_tree = ttk.Treeview(col3, columns=cols, show='headings', height=10)

self.sat_tree.heading("ID", text="ID")

self.sat_tree.column("ID", width=35, anchor="center")

self.sat_tree.heading("Sys", text="Sys")

self.sat_tree.column("Sys", width=35, anchor="center")

self.sat_tree.heading("Freq", text="Freq (MHz)")

self.sat_tree.column("Freq", width=90, anchor="center")

self.sat_tree.heading("Elv", text="Elv")

self.sat_tree.column("Elv", width=35, anchor="center")

self.sat_tree.heading("Az", text="Az")

self.sat_tree.column("Az", width=35, anchor="center")

self.sat_tree.heading("SNR", text="SNR")

self.sat_tree.column("SNR", width=35, anchor="center")

self.sat_tree.pack(fill="both", expand=True)

# Bottom Section

bot_frame = tk.Frame(parent)

bot_frame.pack(fill="both", expand=True, pady=10)

t_frame = tk.LabelFrame(bot_frame, text="Date & Time Log (IST)", padx=5, pady=5)

t_frame.pack(side="left", fill="both", expand=True, padx=(0, 5))

self.txt_time = scrolledtext.ScrolledText(t_frame, height=10, state='disabled', font=("Consolas", 10, "bold"), bg="black", fg="#00ff00")

self.txt_time.pack(fill="both", expand=True)

r_frame = tk.LabelFrame(bot_frame, text="Raw NMEA Stream", padx=5, pady=5)

r_frame.pack(side="left", fill="both", expand=True)

self.txt_raw = scrolledtext.ScrolledText(r_frame, height=10, state='disabled', font=("Consolas", 9), bg="#1e1e1e", fg="white")

self.txt_raw.pack(fill="both", expand=True)

def setup_3d_view(self, parent):

if not HAS_MATPLOTLIB: return

self.fig = Figure(figsize=(5, 5), dpi=100)

self.ax = self.fig.add_subplot(111, projection='3d')

self.ax.set_title("Live Satellite Positions")

self.canvas = FigureCanvasTkAgg(self.fig, master=parent)

self.canvas.get_tk_widget().pack(fill="both", expand=True)

self.animate_3d()

def create_group(self, parent, title, fields):

frame = tk.LabelFrame(parent, text=title, font=("Arial", 11, "bold"), fg="blue")

frame.pack(fill="x", pady=2)

for label, key in fields:

row = tk.Frame(frame)

row.pack(fill="x")

tk.Label(row, text=label+":", width=12, anchor="e", fg="#555").pack(side="left")

val_lbl = tk.Label(row, text="--", font=("Arial", 10, "bold"), anchor="w", fg="black")

val_lbl.pack(side="left", padx=5)

self.labels[key] = val_lbl

def refresh_ports(self):

ports = [p.device for p in serial.tools.list_ports.comports()]

self.port_combo['values'] = ports

if ports: self.port_combo.current(0)

def toggle_connection(self):

if not self.is_running:

try:

self.ser = serial.Serial(self.selected_port.get(), 38400, timeout=1)

self.is_running = True

self.btn_connect.config(text="DISCONNECT", bg="red")

threading.Thread(target=self.read_serial, daemon=True).start()

except Exception as e:

messagebox.showerror("Connection Error", str(e))

else:

self.is_running = False

if self.ser: self.ser.close()

self.btn_connect.config(text="CONNECT", bg="black")

def read_serial(self):

while self.is_running and self.ser:

try:

line = self.ser.readline().decode('utf-8', errors='replace').strip()

if line:

self.parse_nmea(line)

self.log_raw(line)

except:

break

# --- THE MASTER FREQUENCY DECODER ---

def get_frequency_value(self, sys_id, sig_id):

# 1. Handle "Searching" or Empty

if not sig_id or sig_id == '0':

return "Searching..."

# 2. GPS (USA)

if sys_id == "GP":

if sig_id == '1': return "1575.42 MHz" # L1 C/A

if sig_id == '5': return "1227.60 MHz" # L2 CM

if sig_id == '6': return "1227.60 MHz" # L2 CL

if sig_id == '8': return "1176.45 MHz" # L5 (Modern)

# 3. Galileo (Europe)

elif sys_id == "GA":

if sig_id == '7': return "1575.42 MHz" # E1

if sig_id == '1': return "1575.42 MHz" # E1 (Old)

if sig_id == '2': return "1176.45 MHz" # E5a

if sig_id == '8': return "1207.14 MHz" # E5b

if sig_id == '3': return "1278.75 MHz" # E6 (HAS)

# 4. BeiDou (China)

elif sys_id == "GB":

if sig_id == '1': return "1561.09 MHz" # B1I

if sig_id == '3': return "1575.42 MHz" # B1C

if sig_id == '5': return "1176.45 MHz" # B2a (Precision)

if sig_id == '7': return "1207.14 MHz" # B2b

if sig_id == '8': return "1268.52 MHz" # B3I (High Reliability)

# 5. GLONASS (Russia)

elif sys_id == "GL":

if sig_id == '1': return "1602.00 MHz" # L1OF

if sig_id == '3': return "1246.00 MHz" # L2OF

# 6. NavIC (India)

elif sys_id == "GI":

if sig_id == '1': return "1176.45 MHz" # L5

# 7. QZSS (Japan)

elif sys_id == "GQ":

if sig_id == '1': return "1575.42 MHz" # L1

if sig_id == '8': return "1176.45 MHz" # L5

return f"Sig {sig_id}" # Fallback

def parse_nmea(self, line):

parts = line.split(',')

if not parts: return

# 1. RMC (Time, Date, Pos)

if "RMC" in parts[0]:

valid = (len(parts)>2 and parts[2]=='A')

# Time Log (IST Conversion)

time_str, date_str = "--:--:--", "--/--/--"

if len(parts)>1 and parts[1]:

t = parts[1]

try:

h = int(t[0:2])

m = int(t[2:4])

s = int(t[4:6])

# Add 5 hours 30 mins

m += 30

if m >= 60:

m -= 60

h += 1

h += 5

if h >= 24:

h -= 24

time_str = f"{h:02}:{m:02}:{s:02}"

self.update_lbl("time", time_str + " IST", True)

except: pass

if len(parts)>9 and parts[9]:

d = parts[9]

date_str = f"{d[0:2]}/{d[2:4]}/20{d[4:6]}"

self.update_lbl("date", date_str, True)

if time_str != "--:--:--":

self.log_time(f"Time: {time_str} IST | Date: {date_str}")

if valid:

self.update_lbl("lat", f"{parts[3]} {parts[4]}", True)

self.update_lbl("lon", f"{parts[5]} {parts[6]}", True)

self.update_lbl("speed_kn", parts[7], True)

self.update_lbl("course", parts[8], True)

else:

self.update_lbl("lat", "No Fix", False)

# 2. VTG

if "VTG" in parts[0] and len(parts)>7:

self.update_lbl("speed_km", parts[7] + " km/h", True)

# 3. GGA

if "GGA" in parts[0]:

qual = parts[6] if len(parts)>6 else '0'

is_fix = (qual != '0')

self.update_lbl("fix", f"Mode {qual}", is_fix)

self.update_lbl("sats_used", parts[7], is_fix)

self.update_lbl("hdop", parts[8], is_fix)

self.update_lbl("alt", f"{parts[9]} M", is_fix)

self.update_lbl("geoid", f"{parts[11]} M", is_fix)

# 4. GSV (Satellites)

if "GSV" in parts[0]:

try:

sys_id = parts[0][1:3]

# Extract Signal ID (Last field before checksum)

raw_sig_id = parts[-1].split('*')[0]

# Get the frequency string (e.g., "1176.45 MHz")

freq_val = self.get_frequency_value(sys_id, raw_sig_id)

sats_list = []

for i in range(4, len(parts)-1, 4):

if i+3 < len(parts):

sid = parts[i]

elv = parts[i+1]

az = parts[i+2]

snr = parts[i+3].split('*')[0]

if sid and snr:

# Add data to table

sats_list.append((sid, sys_id, freq_val, elv, az, snr))

# Add data to 3D plot

if elv and az:

self.satellites[sid] = (float(az), float(elv), int(snr))

self.root.after(0, self.update_tree, sats_list)

except: pass

# --- GUI UPDATES ---

def update_lbl(self, key, val, status):

self.root.after(0, lambda: self._safe_config(key, val, status))

def _safe_config(self, key, val, status):

if key in self.labels:

color = "#00aa00" if status else "black"

self.labels[key].config(text=val, fg=color)

def log_raw(self, text):

self.root.after(0, lambda: self._append_text(self.txt_raw, text))

def log_time(self, text):

self.root.after(0, lambda: self._append_text(self.txt_time, text))

def _append_text(self, widget, text):

widget.config(state='normal')

widget.insert(tk.END, text + "\n")

widget.see(tk.END)

widget.config(state='disabled')

def update_tree(self, data):

for item in data:

self.sat_tree.insert("", 0, values=item)

children = self.sat_tree.get_children()

# Keep table clean (max 30 rows)

if len(children) > 30:

for c in children[30:]: self.sat_tree.delete(c)

# --- 3D ANIMATION ---

def animate_3d(self):

if not HAS_MATPLOTLIB: return

self.ax.clear()

# Draw Dome

import numpy as np

u = np.linspace(0, 2 * np.pi, 30)

v = np.linspace(0, np.pi / 2, 10)

x_wire = np.outer(np.cos(u), np.sin(v))

y_wire = np.outer(np.sin(u), np.sin(v))

z_wire = np.outer(np.ones(np.size(u)), np.cos(v))

self.ax.plot_wireframe(x_wire, y_wire, z_wire, color='gray', alpha=0.1)

# Compass Labels

self.ax.text(0, 1.1, 0, "N", color='blue')

self.ax.text(0, -1.1, 0, "S", color='blue')

self.ax.text(1.1, 0, 0, "E", color='blue')

self.ax.text(-1.1, 0, 0, "W", color='blue')

# Plot Satellites

for sid, (az, el, snr) in list(self.satellites.items()):

az_rad = math.radians(az)

el_rad = math.radians(el)

# Convert Spherical to Cartesian

x = math.cos(el_rad) * math.sin(az_rad)

y = math.cos(el_rad) * math.cos(az_rad)

z = math.sin(el_rad)

# Color Code

color = 'red'

if snr > 20: color = 'orange'

if snr > 30: color = '#00ff00' # Green

self.ax.scatter(x, y, z, color=color, s=snr*2)

self.ax.plot([x, x], [y, y], [0, z], color='gray', linestyle=':', alpha=0.5)

self.ax.text(x, y, z, sid, fontsize=8)

self.ax.set_axis_off()

self.canvas.draw()

self.root.after(1000, self.animate_3d)

if __name__ == "__main__":

root = tk.Tk()

app = UltimateGPSApp(root)

root.mainloop()